INSTALLATION INSTRUCTIONS

2WD AAM Axle Bracket
1. Position driver side bracket on shock mount so that both end holes align.

2. Mark and drill 3rd hole (furthest from shock) for each axle bracket, as well as the additional vertical holes provided in the bracket .
3. Repeat Steps 1 & 2 for passenger side axle bracket.

4. Hang U-bolts (leg-down) over axle tube inboard of each shock mount – be sure any lines, hoses or wires are on top of U-bolts.

5. Slide guillotine spacers onto U-bolts – again without clamping or crimping any brake lines, wire looms or vent hoses. 

6. Assemble bracket to shock mount using Grade 8 hardware with 1/4” anti-pullout tabs and poly lock nuts inside the front of each shock bracket.
7. Lift U-bolt enough to slide Gr8 washers and thread a nut onto each leg, using the washers to shim the spacer away from the bracket as required.

8. Repeat Steps 6 & 7 for passenger side axle bracket

9.  Assemble nuts & washers to U-bolt legs extending through axle brackets.
10.  Tighten upper nuts to clamp guillotine spacer against axle tube, then   

            tighten lower nuts on U-bolt legs to clamp axle bracket against spacer.
11.  Torque 3 lower Gr8 bolts/nuts in each shock bracket to 55 ft-lbs.

12.  Torque remaining Gr8 hardware on side of brackets to 35 ft-lbs. (LocTite or  equivalent is recommended), and continue with traction bar installation.

LazarSmith©
