INSTALLATION INSTRUCTIONS

Lazar Links
1. Remove sway bar end link nuts, washers & grommets and swing sway bar out of the way.
2. Remove end link nuts from ball studs & drive shafts out of axle tabs.
3. Drill or ream 1/2” taper bore to 1/2” straight bore in axle tabs.

(Note: Drill or ream 2007 and later trucks to 5/8” straight bore.)

4. Assemble end links & hardware (outboard to inboard) in the following order: 1/2” bolt head*SS washer*flanged bushing (2007+ only)*axle tab*flanged bushing (2007+ only)*washer*hex spacer*heim end*hex spacer*washer*double-shear bracket (AAM axle – smaller bracket on passenger side)*SS washer (AAM axle – clipped washer on passenger side)*split-ring lock washer*nut.

5. Position & mark hole(s) in axle tab brackets for double-shear bracket mounting hardware. (Not required on AAM axle driver’s side.)
6. Remove end links & hardware from axle tabs.

7. Drill 3/8” hole(s) into axle tab bracket(s).

8. Repeat Step #4 and assemble 3/8” double-shear bracket hardware to axle tab brackets. (Note: AAM axles use SS blind flag nuts inside brackets.)

9. Torque 3/8” hardware to 50 ft-lbs. & axle tab nuts to 75 ft-lbs.
10.  Locate sway bar ends between urethane grommets on end links

11.  Tighten stainless steel nuts against Grade 8 washers on either side of 

            grommets until they bulge 1/8”.

12.  Tighten stainless steel jam nuts against heim end.

LazarSmith©
