INSTALLATION INSTRUCTIONS

Track Bar Bushings & Hardware
1. Remove mounting bolts & flag nuts from axle and frame brackets.

2. Drop track bar from frame bracket, slide out of axle bracket & remove from truck.

3. If desired, soak bushing shells in penetrating oil & peen non-flanged side towards rubber core.
4. Using appropriately-sized driver & receiver, press bushings out using large hammer, hydraulic press, heavy duty C-clamp or bench vise.

5. Clean track bar bushing bores with emery cloth or sand paper & wipe clean.

6. Insert beveled end of new bushing shell into chamfered side of track bar bushing bore – press about halfway in while guiding true (use plate or heavy washer to evenly distribute load on bushing shell).
            Note: due to track bar bore production tolerances, bushing shells will have      

            varying press-fits; to ease some installations, the split shell may be  

            longitudinally clamped in a vise to allow sawing through the shell’s  

            seam to remove a portion – usually, a hacksaw’s kerf width is sufficient.

7. Insert bushing core into shell (with correct orientation if offset - hole should be located towards the end of track bar) & press into track bar until protrusion is about equal on both sides.

8. Drill 3/16” diameter hole 3/4” deep through top of each track bar end into bushing core and insert stainless steel spring pins (Loctite, silicone or adhesive may be used on pins if hole is drilled oversized).
9. Reinstall track bar axle side first, using a screwdriver to hang frame end bracket.

10.  Insert short track bar bolt through axle bracket (AB) & toothed washers (TW) into  

             flag nut in the following order: TW*AB*TW*bushing*TW*AB*TW.
             Be sure to avoid cross-threading the nut – use the flag for positioning and apply   

             Loctite to flag nut threads.
             Note: bolt end should not contact axle tube when torqued to specification –   

             use 1/2” Gr8 washer under bolt head if necessary.
11.  Align driver’s side holes & insert long track bar bolt through frame bracket (FB)  

             in the same order: TW*FB*TW*bushing*TW*FB*TW – use Loctite on threads.

12.  Torque track bar bolts to at least 155 ft-lbs. & adjust drag link as needed.
LazarSmith©
