INSTALLATION INSTRUCTIONS

Traction Blocks
1. Remove leaf spring U-bolts & nuts, and axle saddle clamps if present.
2. Remove leaf pack top plates if installing 3/4” U-bolts.

3. Raise frame of truck over real axle enough to remove lift blocks and/or spacer plates from spring perch. (Leaf pack pin bolts may need to be removed.)
4. Place left & right traction blocks on axle tube spring perches with heim mounts located forward & inboard. (Replace leaf pack pin bolts if needed.)
5. Lower frame until leaf packs rests on traction blocks.
6. Place top plates on leaf packs if using 3/4” U-bolts.

7. Assemble U-bolts & nuts to leaf packs & axle (saddle clamps if present).
8. If using 3/4” U-bolts, insert from bottom – thread washers & nuts from top.
9. Locate final position of traction blocks and lower frame completely on axle.

10.  Jounce suspension to settle components & check positioning.

11.  Torque OEM U-bolt nuts to 110 ft-lbs.

12.  Torque 3/4” U-bolts to 175 ft-lbs.
LazarSmith©
