INSTALLATION INSTRUCTIONS

Universal Axle Brackets
1. Clean axle tubes of any dirt, grease or rust.
2. Hang U-bolts over axle tubes (2 each between differential & shock brackets) – do not lay on brake line, wire harness, vent tube, etc.

3. Thread 1 nut fully on to each U-bolt leg before lubricating threads.

4. Place 1 flat washer on each U-bolt leg, slide bracket onto U-bolt, followed by another flat washer, lock washer and nut.

5. Repeat Step 3 for other U-bolt, then Steps 3 & 4 for other bracket.

6. Space both brackets approximately equidistant from differential housing.

7. Hand-tighten all lower nuts and swing both brackets into position – pointing forward & down about 45 degrees.
8. Torque lower nuts to 50 ft-lbs. and insert 3/4” bolts through axle brackets.
9. Place wooden block or other suitable striking surface onto 3/4” bolt and set bracket against axle tube with large hammer – repeat for other side.
10.  Remove 3/4” bolts, check nut torque and continue installing traction bars.
11.  After test driving, re-torque lower nuts if required and tighten upper nuts 
            against flat washers.
12.  Note: brackets and/or U-bolts can be welded to axle tubes if needed (1” of 
       weld each is sufficient).
LazarSmith©
