INSTALLATION INSTRUCTIONS

Tow Hook Xtenders

- Remove OEM hardware holding OEM tow hooks to frame.

- Remove tow hooks from truck.

- Lubricate OEM hardware threads.

- Position Xtenders within bumper.

- Assemble OEM hardware through Xtenders to frame in the following order: lower outboard*upper inboard*lower inboard*upper outboard.

- Lightly tighten two of the OEM nuts.

- Level Xtenders within bumper.

- Torque OEM hardware to 50 ft-lbs. in the following order: upper inboard*lower inboard*lower outboard*upper outboard.

- Assemble tow hooks to Xtenders with stainless steel hardware.

- Thread stainless steel bolts against split-ring lock washers.

- Position tow hooks on Xtenders level with each other.

- Torque stainless steel hardware to 25 ft-lbs.

LazarSmith©